# Portable Floor Model Deep Fryer

## Lighting instructions

- The safety valve should be closed at least 5 minutes for any unburned gas to escape the combustion chamber.

- Turn the thermostat to the slowest possible setting.

- Turn control gas dial to the pilot position.

- Depress and hold reset button while lighting the pilot, hold for about 45 seconds.

- Release the reset button. The pilot should stay lit.

- Turn the control gas button to the on position.

- Set thermostat dial to the desired temperature.  The main burners should light.

Specifications

- Fill the oil tank with 8 to 10 gallons of oil.

- A 20 lb. Propane tank (120,000 btu) will provide approximately 4 hours of cooking time.

- The two cooking baskets are 4” deep x 12” long x 5” wide.

- The entire unit will be extremely hot during cooking.

### Shutdown instructions

- Turn thermostat to lowest possible setting.

- Slightly lift the lock button and turn the control gas dial to the off position.

- Allow oil to cool.

- Drain oil into a suitable container.

- Wipe excess oil from the unit.

# Caution: Oil must be drained from fryer before moving. Moving fryer filled with oil may case personal injury, spills or burning.

**If you need assistance call our 24 hour emergency service at (330) 753-9104**