Mushroom Heater Assembly and Lighting Instructions

Assembly

Step1:  Attach legs to control cone.  Depress clutch spring on leg and slide leg into leg holder on control cone.  Important: Make sure clutch spring locks into hole in leg holder.

Step 2:  Attach Reflector.  Place centre hole of reflector over folding wing nut and slide reflector onto emitter.  Unfold the folding wing nut and screw down onto the reflector until reflector is held firmly onto emitter.  Important: over tightening the folding wing nut can damage the reflector.

Step 3:  Hang Propane tank and attach skirt.  Hang the propane cylinder from the hook beneath the control cone and between the legs.  Wrap the skirt around the legs. Secure with Velcro at top. Zip up. Move skirt so that zip runs down one of the legs.  Important: Cylinder adds important weight to the Zubri heater and helps to keep it stable.

Lighting the Heater

Step 1: Turn on gas supply from propane cylinder

Step 2: Hold flame under burner head, close to pilot light igniter.

Step 3: Start gas flow by pushing in control knob and turning counterclockwise 90 degrees.

Gas will ignite

Important: Keep control knob pushed in for at least 15 seconds.  If flame goes out for any reason wait 2 minutes and repeat steps 2 & 3.

Heater will heat an area 18’-20’ wide and will run on high for six hours on a single 20lbs. propane tank.