Popcorn Machines

Operational Tips

Switches

Light and Warmer- Turns on heat lamp and optional strip heater under corn pan

Kettle Motor Switch- turns on motor that drives kettle agitator shaft.

Kettle Heat Switch- turns on heat element inside kettle.

To Pop

- Turn on kettle heat switch and kettle motor and exhaust switch.

- “Test pop” a kettle of corn: place oil measure in kettle with three corn kernels; when kernels pop, place remainder of oil, measure corn and flavoring in kettle; close lid.

- When corn has popped, rotate the handle to empty popcorn from kettle; return kettle to upright position.

- Repeat the cycle; subsequent batches are always more flavorful than the initial batch.

- Turn off the kettle heat switch on the final batch when popcorn breaks over the lid of the kettle to eliminate smoke and odor.

- Turn off kettle heat and kettle motor and exhaust switches.

- Wipe exterior of kettle with heavy cloth after cooling of kettle to prevent burn or stain of oil drippings.

- Turn off all switches.

To Clean

- Allow Kettle to cool until warm enough to handle.

- Unplug kettle and remove from machine.

- Wipe kettle, kettle lids, cross bar and exterior clean.  DO NOT IMMERSE KETTLE IN WATER.

- Remove popped corn and pack in bags.

- Remove the “old maid” pan, empty un-popped kernels and wipe clean.

- Wipe interior of machine clean.

869 W. Waterloo Rd. Akron, OH 44314   330-753-9104

www.millersparty.com